مشخصات فني استعلام شماره : RKS – 8931122 – KM/SA

| مقدار | شــــــرح كالا | واحد | شماره طبقه بندي | شماره قلم |
| --- | --- | --- | --- | --- |
|  | BUSHINGS ,CARBOON STEEL ,SEAMLESS,SCREWED , API LINE PIPE THREAD , HEXAGONAL ONE END MAT : IN ACC . WITH ASTM – A105 DIMENSIONS , MARKING AND TOLERANCES TO ASME B16 . 11 OR BS 3799 FOR USE WITH SCREWED FITTINGS OF ANY OF THE 2000 , 3000 & 6000 LBS CLASSES . |  |  |  |
| 50 | 3 / 8 × 1 / 4 IN . | NO | 7605140132 | 01 |
|  | COUPLINGS ,CARBON STEEL , SEAMLESS,SCREWED , API LINE PIPE THREAD MAT : IN ACC . WITH API STD . 5L GRADE B DIMENSIONS AND PRESSURE RATING , TO API STD . 5L |  |  |  |
| 40 | 3 / 8 IN . | NO | 7605280062 | 02 |
|  | COUPLINGS , CARBON STEEL , SEAMLESS, SCREWED API LINE PIPE THREAD MAT : IN ACC . WITH ASTM – A105 DIMENSIONS, MARKING AND TOLERANCES TO ANS B 16 . 11 |  |  |  |
| 400 | 3 / 4 IN . CWP 3000 LBS | NO | 7605281122 | 03 |
| 10 | 2 IN . CWP 6000 LBS | NO | 7605282202 | 04 |
|  | NIPPLES , LINE PIPE , SEAMLESS , BLACK , SCREWED API LINE PIPE THREAD MAT : ELECTRIC FURNACE , OPEN HEARTH , OR BASIC OXYGEN STEEL IN ACC . WITH API STD . 5L GRADE B, ASTM–A 106 GRADE B OR DIN 17175 ST 45.8 – 1 |  |  |  |
| 500 | 1 / 2 ×4 IN . SCHED 40 | NO | 7605560822 | 05 |
| 500 | 1 / 2 × 6 IN . SCHED 40 | NO | 7605560842 | 06 |
|  | NIPPLES , SWEDGE , CONCENTRIC , SEAMLESS BLACK , ENDS SCREWED API LINE PIPE THREAD MAT : ELECTRIC FURNACE , OPEN HEARTH , OR BASIC OXYGEN STEEL IN ACC . WITH API STD . 5L GRADE B , ASTM –A 106 GRADE B OR DIN 17175 ST 45.8.1 |  |  |  |
| 30 | 1 × 3 / 4 IN SCHED 40 | NO | 7605580322 | 07 |
| 10 | 3 / 4 × 1 / 2 IN . SCHED 80 | NO | 7605581222 | 08 |
|  | PLUGS , BULL , SOLID , SCREWED API LINE PIPE THREAD , CWP 6000 PSI . NON – SHOCK MAT : CARBON STEEL IN ACC . ASTM – A105 DIMENSIONS , MARKING AND DESIGN TO BS 3799 |  |  |  |
| 20 | 1 IN . | NO | 7605662162 | 09 |
|  | ELBOWS , 90 DEG . CARBON STEEL , GALVANIZED SEAMLESS , SCREWED API LINE PIPE THREAD , MAT : IN ACC . WITH ASTM A – 105 DIMENSIONS , MARKING AND TOLERANCES TO ASME B 16 . 11 |  |  |  |
| 100 | 2 IN . CWP 3000 LBS | NO | 7606391202 | 10 |
|  | NIPPLES , CARBON STEEL , GALVANIZED , HEXAGONAL SEAMLESS , DOUBLE MALE , SCREWED API LINE PIPE THREAD MATERIAL TO ASTM A – 105 DIMENSIONS , MARKING AND TOLERANCES TO ANS B . 16 . 11 |  |  |  |
| 300 | 1 /2 IN . CWP 3000 LBS | NO | 7606541082 | 11 |
|  | NIPPLES , LINE PIPE , SEAMLESS , GALVANIZED SCREWED API LINE PIPE THREAD MAT : ELECTRIC FURNACE , OPEN HEARTH OR BASIC OXYGEN STEEL IN ACC. WITH API STD . 5L GRADE B , ASTM – A106 GRADE B OR DIN 17175 ST 48.8.1 SIZE LENGTH |  |  |  |
| 300 | 1 / 2×3 IN . SCHED 40 | NO | 7606560802 | 12 |
| 200 | 1 / 2× 4 IN . SCHED 40 | NO | 7606560822 | 13 |